##### DRAFT EAST AFRICAN STANDARD

Automotive gas oil (Automotive diesel)— Specification

EAST AFRICAN COMMUNITY

Copyright notice

This EAC document is copyright-protected by EAC. While the reproduction of this document by participants in the EAC standards development process is permitted without prior permission from EAC, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from EAC.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to EAC’s member body in the country of the requester:

© East African Community 2023 — All rights reserved

East African Community

P.O. Box 1096,

Arusha

Tanzania

Tel: + 255 27 2162100

Fax: + 255 27 2162190

E-mail: eac@eachq.org

Web: www.eac-quality.net

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement. Violators may be prosecuted.

Foreword

Development of the East African Standards has been necessitated by the need for harmonizing requirements governing quality of products and services in the East African Community. It is envisaged that through harmonized standardization, trade barriers that are encountered when goods and services are exchanged within the Community will be removed.

The Community has established an East African Standards Committee (EASC) mandated to develop and issue East African Standards (EAS). The Committee is composed of representatives of the National Standards Bodies in Partner States, together with the representatives from the public and private sector organizations in the community.

East African Standards are developed through Technical Committees that are representative of key stakeholders including government, academia, consumer groups, private sector and other interested parties. Draft East African Standards are circulated to stakeholders through the National Standards Bodies in the Partner States. The comments received are discussed and incorporated before finalization of standards, in accordance with the Principles and procedures for development of East African Standards.

East African Standards are subject to review, to keep pace with technological advances. Users of the East African Standards are therefore expected to ensure that they always have the latest versions of the standards they are implementing.

The committee responsible for this document is Technical Committee EASC/TC 068, Petroleum and petroleum products.

Attention is drawn to the possibility that some of the elements of this document may be subject of patent rights. EAC shall not be held responsible for identifying any or all such patent rights.

This second edition cancels and replaces the first edition (EAS 158:2019) which has been technically revised.

Automotive gas oil (Automotive diesel)— Specification

# 1	Scope

This Draft East African Standard specifies requirements, sampling and test methods for Automotive Gas Oil (AGO), commonly known as diesel used to power internal combustion engines

The standard applies to automotive gas oil (automotive diesel ) as manufactured, stored, transported and marketed

# 2	Normative references

The following referenced documents referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ASTM D86, Standard Test Method for Distillation of Petroleum Products at Atmospheric Pressure

ASTM D93, Standard Test Methods for Flash Point by Pensky-Martens Closed Cup Tester

ASTM D95, Standard Test Method for Water in Petroleum Products and Bituminous Materials by Distillation

ASTM D130, Standard Test Method for Corrosiveness to Copper from Petroleum Products by Copper Strip Test

ASTM D189, Standard Test Method for Conradson Carbon Residue of Petroleum Products

ASTM D473, Standard Test Method for Sediment in Crude Oils and Fuel Oils by the Extraction Method

ASTM D482, Standard Test Method for Ash from Petroleum Products

ASTM D 664, Standard Test Method for Acid Number of Petroleum Products by Potentiometric Titration

ASTM D974, Standard Test Method for Acid and Base Number by Colour-Indicator Titration

ASTM D976, Standard Test Method for Calculated Cetane Index of Distillate Fuels

ASTM D1298, Standard Test Method for Density, Relative Density, or API Gravity of Crude Petroleum and Liquid Petroleum Products by Hydrometer Method

ASTM D1500, Standard Test Method for ASTM Colour of Petroleum Products (ASTM Color Scale)

ASTM D1552, Standard Test Method for Sulphur in Petroleum Products (High-Temperature Method)

ASTM D2500 [IP 309], Standard test method for cloud point of petroleum products

ASTM D6078, Standard Test Method for Evaluating Lubricity of Diesel Fuels by the Scuffing Load Ball-onCylinder Lubricity Evaluator (SLBOCLE)

ASTM D4052, Standard Test Method for Density, Relative Density, and API Gravity of Liquids by Digital Density Meter

1

ASTM D4294,  Standard Test Method for Sulfur in Petroleum and Petroleum Products by Energy Dispersive X-ray Fluorescence Spectrometry

ASTM D5453, Standard Test Method for Determination of Total Sulfur in Light Hydrocarbons, Spark Ignition Engine Fuel, Diesel Engine Fuel, and Engine Oil by Ultraviolet Fluorescence

ASTM D 6304, Standard Test Method for Determination of Water in Petroleum Products, Lubricating Oils, and Additives by Coulometric Karl Fischer Titration

ASTM D7042, Standard Test Method for Dynamic Viscosity and Density of Liquids by Stabinger Viscometer (and the Calculation of Kinematic Viscosity)

EN 12916, Petroleum products — Determination of aromatic hydrocarbon types in middle distillates — High performance liquid chromatography method with refractive index detection

IP 13, Petroleum products ― Determination of carbon residue ― Conradson method

IP 34, Determination of flash point ― Pensky-Martens closed cup method

IP 53, Crude petroleum and fuel oils ― Determination of sediment ― Extraction Method

IP 71 Sect 1, Petroleum products ― Transparent and opaque liquids ― Determination of kinematic viscosity and calculation of dynamic viscosity

IP 74, Petroleum products and bituminous materials ― Determination of water ― Distillation method

IP 123, Petroleum products ― Determination of distillation characteristics at atmospheric pressure

IP 154, Petroleum products ― Corrosiveness to copper ― Copper strip test

IP 160, Crude petroleum and liquid petroleum products ― Laboratory determination of density ― Hydrometer method

ISO 2049, Petroleum products — Determination of colour (ASTM scale)

ISO 2160, Petroleum products — Corrosiveness to copper — Copper strip test

ISO 2719, Determination of flash point — Pensky-Martens closed cup method

ISO 3015, Petroleum products — Determination of cloud point

ISO 3104, Petroleum products — Transparent and opaque liquids — Determination of kinematic viscosity and calculation of dynamic viscosity

ISO 3170, Petroleum liquids — Manual sampling

ISO 3171, Petroleum liquids — Automatic pipeline sampling

ISO 3405, Petroleum products — Determination of distillation characteristics at atmospheric pressure

ISO 3675, Crude petroleum and liquid petroleum products — Laboratory determination of density — Hydrometer method

ISO 3733, Petroleum products and bituminous materials — Determination of water — Distillation method

ISO 3735, Crude petroleum and fuel oils — Determination of sediment — Extraction method

ISO 4259, Petroleum products — Determination and application of precision data in relation to methods of test ISO 4264, Petroleum products — Calculation of cetane index of middle-distillate fuels by the four-variable equation

ISO 5165, Petroleum products — Determination of the ignition quality of diesel fuels — Cetane engine method

ISO 6245, Petroleum products — Determination of ash

ISO 6615, Petroleum products — Determination of carbon residue — Conradson method

ISO 6619, Petroleum products and lubricants — Neutralization number — Potentiometric titration method

ISO 7537, Petroleum products — Determination of acid number — Semi-micro colour-indicator titration method

ISO 10370, Petroleum products — Determination of carbon residue (micro method)

ISO 12156-1, Diesel fuels — Assessment of lubricity by HFRR

ISO 12185, Crude petroleum and petroleum products — Determination of density — Oscillating U-tube method

ISO 12205, Petroleum products — Determination of the oxidation stability of distillate fuels

ISO 12937, Petroleum products — Determination of water — Coulometric Karl Fisher titration method

ISO 20846, Petroleum products — Determination of sulfur content of automotive fuels — Ultraviolet fluorescence method

ISO 20847, Petroleum products — Determination of sulfur content of automotive fuels — Energy dispersive Xray fluorescence spectrometry

ISO 20884, Petroleum products — Determination of sulfur content of automotive fuels — Wavelengthdispersive X-ray fluorescence spectrometry

ASTM D4176, Standard Test Method for Free Water and Particulate Contamination in Distillate Fuels (Visual Inspection Procedures)

# 3	Terms and definitions

For the purposes of this document, the following terms and definitions apply.

ISO and IEC maintain terminological databases for use in standardization at the following addresses:

ISO Online browsing platform: available at http://www.iso.org/obp

3.1

automotive gas oil (AGO) fuel that is used in high-speed diesel engines

##### 3.2 additive

material added to AGO usually in small amounts, to impart or enhance desirable properties or to suppress undesirable properties

3.3

##### dyes

chemicals added to fuel for visual identification.

###### 3.4 markers

chemical substances introduced into automotive gas oil for identification, traceability and regulation without altering their properties.

3.5

polycyclicaromatic hydrocarbon

total aromatic hydrocarbon content less the mono-aromatic hydrocarbon content

3.6

portable containers

freight containers designed for intermodal transportation that is; land, sea, or air. There are various dimensions and classifications for these containers, making them interchangeable and stackable for efficient global trade.

# 4	Requirements

## 4.1	General requirements

4.1.1 Automotive gas oil shall be a homogeneous blend of hydrocarbons derived from petroleum refining.

4.1.2 Automotive gas oil shall be free from inorganic acids, suspended matter, and shall remain uniform in normal storage conditions

##### 4.2 Specific quality requirements

Automotive diesel fuel shall comply with the limits specified in Table 1, when tested in accordance with the test methods specified therein.

Table 1 —Specific Requirements for  Automotive Gas Oil (Automotive diesel)

| S/ No. | Property | Limit |  | Test method )a |
| --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum |  |
|  | Appearance | clear and Bright |  | ASTM D4176 |
|  | Cetane number b) | 51.0 | — | ISO 5165 |
|  | Cetane index | 48.0 | — | ISO 4264 ASTM D976 |
|  | Density at 15 C c), kg/m3 | 820 | 870 | ISO 3675 ISO 12185 ASTM D1298 ASTM D4052 IP 160 |
|  | Density at 20 C | 817 | 867 |  |
|  | ASTM colour | — | 3.5 | ISO 2049 ASTM D1500 |
|  | Polycyclic aromatic hydrocarbons, % (v/v) | — | 11 | EN 12916 |
|  | Sulphur content d), mg/kg | — | 10.0 | ISO 20884 ASTM D1552 ASTM D4294 ASTM D5453 |
|  | Flash point, C | 60 | — | ISO 2719 ASTM D93 IP 34 |
|  | Carbon residue (on 10 % distillation residue), % (m/m) | — | 0.15 | ISO 10370 ISO 6615 ASTM D189 IP 13 |

|  |  | Ash content, % (m/m) | — | 0.01 | ISO 6245 ASTM D482 |
| --- | --- | --- | --- | --- | --- |
|  |  | Water content, mg/kg | — | 200 | ISO 12937 ASTM D95 IP 74 ISO 3733 ASTM D 6304 |
|  |  | Copper strip corrosion (3 h at 50 ºC), rating | Class 1 |  | ISO 2160 ASTM D130 IP 154 |
|  |  | Oxidation stability, g/m3 | — | 25 | ISO 12205 |
|  |  | Lubricity, corrected wear scar diameter (wsd 1.4) at 60 ºC, m | — | 450 | ISO 12156-1 ASTM D6078 |
|  |  | Viscosity at 40 ºC, mm2/s | 2.0 | 5.3 | ISO 3104 ASTM D445 ASTM D7042 IP 71 |
|  |  | Cloud point, ºC | To be reported |  | ISO 3015 ASTM D2500 |
|  |  | Cold filter plugging point (CFPP), ºC | — | 12 | IP 309 |
|  |  | Sediment, % m/m | — | 0.01 | ISO 3735 ASTM D473 IP 53 |
|  |  | Neutralization value: Strong acid No., KOH, mg/g Total acid No., KOH, mg/g |  | Nil 0.5 | ISO 6619 ASTM D974 ISO 7537 ASTM D 664 |
| - ) |  | Distillation e |  |  | ISO 3405 ASTM D86 IP 123 |
|  |  | • Initial boiling point, ºC | To be reported | To be reported |  |
|  |  | 10% (v/v) recovered at ºC | To be reported | To be reported |  |
|  |  | 50 % (v/v) recovered at ºC | To be reported | To be reported |  |
|  |  | • % (V/V) recovered at 250 ºC | - | 65 |  |
|  |  | • % (V/V) recovered at 350 ºC | 85 | - |  |
|  |  | • 95% (V/V) recovered at ºC | - | 363 |  |
|  |  | • Final boiling point, ºC | - | 400 |  |
| a | See also 7.1. |  |  |  |  |
| b | See also 7.4. |  |  |  |  |
| c | See also 7.2. |  |  |  |  |
| d | See also 7.3. |  |  |  |  |
| e | For the calculation of the cetane index the 10 %, 50 % and 90 % (V/V) recovery points are also needed. |  |  |  |  |

#### 5 Dyes and markers

5.1 	The use of dyes and markers is allowed.

5.2 The dye content shall be reported and shall not alter any parameters of automotive gas oil  out of the specified range as indicated under Table 1.

#### 6 Additives

#### In order to improve the performance quality, the use of additives is allowed. Suitable fuel additives without known harmful side-effects are recommended in the appropriate amount, to help to avoid deterioration of drivability and emissions control durability.7 Precision and dispute

7.1 All test methods referred to in this East African Standard include a precision statement. In cases of dispute, the procedures for resolving the dispute and interpretation of the results based on test method precision, described in ISO 4259, shall be used.

7.2 	In cases of dispute concerning density, ISO 3675 shall be used.

7.3 	In cases of dispute concerning sulfur content, ISO 20847 is unsuitable as an arbitration method.

7.4 For the determination of cetane number alternative methods may also be used in cases of dispute, provided that these methods originate from a recognized method series, and have a valid precision statement, derived in accordance with ISO 4259, which demonstrates precision at least equal to that of the referenced method. The test result, when using an alternative method, shall also have a demonstrable relationship to the result obtained when using the referenced method.

#### 8 Packaging

The condition of the containers, rail tankers and road tank vehicles shall be such as not to be detrimental to the quality of the fuel during normal transportation and storage. The containers shall be acceptably sealed or leak proof, clean, and free from materials soluble in diesel.

##### 9 Labelling

9.1 The following information shall appear in legible and indelible marked on the portable container, or on a label affixed to the portable  container

- the supplier’s and receiver’s name and physical address;

- the name “AUTOMOTIVE GASOIL” or “DIESEL”, or “GASOIL”;

- the quantity in litres or metric tonnes;

- the batch number;

- the words “FLAMMABLE MATERIAL”; and

- the warning “DANGER AUTOMOTIVE GASOIL” or “DANGER DIESEL”, or “DANGER GASOIL”

g. country of origin

9.2 For bulk transportation the above information shall be in the documentation accompanying the product and the respective container shall be marked with;

a. the words “FLAMMABLE MATERIAL”; and

b)	the warning “dangerous goods”

9.3 Stickers shall be used in labelling in case of vehicles which have compartments and transport more than one type of fuels.

# 7	Sampling

Sampling shall be done in accordance with ASTM D4057 or ASTM D4177.

Bibliography

EAS 177:2019, Automotive Gas Oil — Specification